# IoT-Based Carbon Capture and CO₂ Curing System for Sustainable Concrete Production

## Program

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

#define MQ135_IN 34

#define MQ135_OUT 35

#define FAN_RELAY 26

#define VALVE_RELAY 27

#define FLOW_SENSOR 32

volatile int flowPulse = 0;

float flowRate = 0;

void IRAM_ATTR flowInterrupt() {

flowPulse++;

}

void setup() {

Serial.begin(115200);

pinMode(FAN_RELAY, OUTPUT);

pinMode(VALVE_RELAY, OUTPUT);

pinMode(FLOW_SENSOR, INPUT);

attachInterrupt(digitalPinToInterrupt(FLOW_SENSOR), flowInterrupt, RISING);

digitalWrite(FAN_RELAY, LOW);

digitalWrite(VALVE_RELAY, LOW);

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println("OLED not found");

while (true);

}

display.clearDisplay();

display.setTextColor(WHITE);

}

void loop() {

int sensorIn = analogRead(MQ135_IN);

int sensorOut = analogRead(MQ135_OUT);

float inputCO2 = map(sensorIn, 0, 4095, 400, 5000);

float outputCO2 = map(sensorOut, 0, 4095, 400, 5000);

digitalWrite(FAN_RELAY, HIGH);

digitalWrite(VALVE_RELAY, HIGH);

flowRate = flowPulse * 0.1;

flowPulse = 0;

Serial.print("Input CO2: ");

Serial.print(inputCO2);

Serial.println(" ppm");

Serial.print("Output CO2: ");

Serial.print(outputCO2);

Serial.println(" ppm");

Serial.print("Flow Rate: ");

Serial.print(flowRate);

Serial.println(" L/min");

Serial.println("-------------------------");

display.clearDisplay();

display.setTextSize(1);

display.setCursor(20, 0);

display.println("CO2 MONITOR");

display.setCursor(0, 15);

display.print("In : ");

display.print(inputCO2, 0);

display.println(" ppm");

display.setCursor(0, 30);

display.print("Out: ");

display.print(outputCO2, 0);

display.println(" ppm");

display.setCursor(0, 45);

display.print("Flow:");

display.print(flowRate);

display.println(" L/m");

display.setCursor(0, 55);

if (outputCO2 < inputCO2) {

display.print("CO2 Reduced");

} else {

display.print("No Change");

}

display.display();

delay(2000);

}